深圳[image: image5.bmp]市华利进电子有限公司

地址：深圳市光明新区公明楼村第二工业区中泰街六栋

电话：0755-83956780     传真：0755-83956781

Http//www.szhlxdz.com     E-mail:hlx_8@163.com

承认书

客    户：
客户料号：
品    名： HLSM063-4R7M
拟    制：   赖  富  忠
审    核：   刘  素  容
批    准：   廖  尊  祥
日    期：   2012-5-7
	客 户 承 认
	日 期

	
	


深圳华利进电子有限公司

规格书

SPECIFICATION   FOR  APPROVAL

	SPECIFICATION FOR
	DATE:    2012/5/7

	P/N:             HLSM063-4R7M
	ISSUE NO:  EG12050702

	1． DIMENSION  (UNIT:mm)

 SHAPE \* MERGEFORMAT 




	A：    7.8Max
	B：  6.8 ± 0.3
	C：  3.0 Max
	D：  2.8 ± 0.3

	E：    1.6 ± 0.3
	F：  +0.20 /+0 
	G：  
	H：

	2．ELECTRICAL  REQUIREMENTS

	TEST CONDITION
	TEST  FREQ/V
	TEST INSTRUMENTS

	L
	4.7μH±20%
	＠100KHz/0.1V
	LCR-METER
	CH-1062/VR115/116

	DCR
	44mΩ (Max)
	
	DCR-METER
	VR131

	IDC
	6Amp
	
	DC POWER SUPPLY
	DH1716-2D

	ISAT
	6AmP
	  100KHz/0.1V
	CURENT SOURCE
	VR7210+VR116

	3．NOTE：      SGS: No.CANEC0903405304

	3-1.All test data is referenced to 25℃ ambient

	3-2. IDC: DC current(A) that will cause an approximate ⊿T of 40℃

	3-3. ISAT:DC current(A) that will cause L0 to drop approximately 30%

	3-4. Use Temperature:  Minimum Temperature -25℃ Ambient temperature of inductor.  Maximum Temperature +125℃ The value of temperature is including ambient and temperature rise of inductor. 

	3-5. Storage Condition: Temperature 25 to 35℃, Humidity 45 to 80 RH

	4．APPROVED  BY  CUSTOMER：
	DRAWN  BY
	CHECKED  BY
	APPROVED  BY

	
	赖 富 忠
	刘 素 容
	廖 尊 祥


EG：01                                                                      REV：00

深圳华利进电子有限公司
检验记录表

	品名
	HLSM063-4R7M
	订单号码
	

	图号
	EG12050702
	检验日期
	2012/5/7

	批量
	
	抽样依据
	MIL-STD-105E质量水平：II

	测试仪器
	LCR: VR116  DCR:VR131

DC Current Sourec:VR7210
	测试条件
	＠100kHz/0.1V

	检验内容
	电参数
	外形尺寸

	检验项目
	L（μH）
	DCR

（mΩ）
	A

(mm)
	B

(mm)
	C

(mm)
	D

(mm)
	E

(mm)
	F

(mm)

	
	0A
	6A
	
	
	
	
	
	
	

	要求
	4.7±20%
	80%L（0）TYP
	44Max
	7.8Max
	6.8±0.3
	3.0Max
	2.8±0.3
	1.6±0.3
	+0.20/+0

	1
	4.62
	3.79
	82.1%
	38.6
	7.27
	6.79
	2.75
	2.64
	1.41
	0.12

	2
	4.93
	4.00
	81.3%
	39.1
	7.30
	6.79
	2.76
	2.86
	1.57
	0.10

	3
	4.76
	3.89
	81.8%
	38.9
	7.28
	6.77
	2.78
	2.67
	1.52
	0.11

	4
	4.56
	3.68
	80.9%
	38.4
	7.31
	6.80
	2.82
	2.70
	1.49
	0.09

	5
	4.81
	3.88
	80.8%
	39.6
	7.29
	6.77
	2.84
	2.68
	1.51
	0.14

	6
	4.87
	3.94
	81.1%
	40.1
	7.31
	6.76
	2.75
	2.69
	1.48
	0.13

	7
	4.34
	3.52
	81.3%
	38.7
	7.29
	6.79
	2.88
	2.66
	1.68
	0.07

	8
	4.91
	3.97
	80.9%
	39.7
	7.30
	6.78
	2.86
	2.61
	1.66
	0.12

	9
	4.77
	3.90
	81.8%
	38.2
	7.31
	6.79
	2.77
	2.72
	1.55
	0.13

	10
	4.68
	3.78
	80.8%
	38.6
	7.28
	6.76
	2.81
	2.83
	1.61
	0.09

	Ｘ
	
	
	
	
	
	
	
	
	
	

	R
	
	
	
	
	
	
	
	
	
	

	 SHAPE \* MERGEFORMAT 



	决议：
	QA:

  钟  建  美

	
	
	主管:

唐  彩  玉

	
	
	批准: 

  廖  尊  祥

	
	
	日期：

2012/5/7


QC.01                                                                                REV:00

深圳华利进电子有限公司
检验记录

	品名
	HLSM063-4R7M
	订单号码
	

	图号
	EG12050702
	检验日期
	2012/5/7

	批量
	
	抽样依据
	MIL-STD-105E质量水平：II

	测试仪器
	LCR: VR116

DC Current Sourec: VR7210

DC Power supply: DC 1716-2D    
	测试条件
	＠100kHz/0.1V

	CURRENT VS INDUCTANCE

    

     



L

ΔT℃





                  0         1       2        3       4       5        6       7

                                            DC  CURRENT（A）



	备注：

     

	检测：  钟 建 美
	审核：唐 彩 玉
	复核：  廖 尊 祥


卷装：(R)                         卷带尺寸
 SHAPE \* MERGEFORMAT 



                               卷轮尺寸

 SHAPE \* MERGEFORMAT 



�





2





1





8.4





4R7











B





E





F





C





A





D





3.7





3.5





B





E





F





C





A





D





决议：





（μH）


6.0





5.0





4.0





3.0





2.0





1.0





60





50





40





30





20





10





0





TEMP RISE（℃）





INDUCTANCE





3.6±0.15





7.2±0.1





8.3±0.1





ø1.5±0.1





4.0±0.1





2.0±0.1





12.0±0.1





1.75±0.1





7.5±0.1





16.0±0.3





0.3±0.1





C





2.0±0.5





Ø20.0±0.5





Ø13.0±1.0





Ø100.0±1.0





Ø330.0±2.0





16.0±0.5





2.0±0.2





每卷数量：1000Pcs








